SPACEMIT K3-CoM260 Series Modules

8-CORE X100™ 64-bit RISC-V CPU + 8-CORE A100™ AI CPU+ LPDDR5

Data Sheet

Document History

| Version | Date | Description of Change |
| --- | --- | --- |
| v0.1 | 2025/12/09 | - Initial preliminary release. |

Table of Contents

Chapter 1.	Overview	1

Chapter 2.	Functional Description	4

  - Ampere GPU	4

    - Compute Features	5

    - Graphic Features	6

  - Cortex CPU Complex	6

  - Memory Subsystem	7

  - Memory	8

  - Video Input Interfaces	9

    - MIPI Camera Serial Interface (CSI)	9

    - Video Input (VI)	11

    - Image Signal Processor (ISP)	11

  - Sensor Processing Engine	12

  - Security Subsystem	12

    - Platform Security Controller	12

    - Security Engine	13

  - Display Controller	13

  - High-Definition Audio-Video Subsystem	15

    - Multi-Standard Video Decoder	15

    - Video Image Compositor (VIC)	16

    - Audio Processing Engine (APE)	16

    - High-Definition Audio (HDA)	17

  - Interface Descriptions	18

    - Universal Serial Bus (USB)	18

      - USB 2.0 Operation	18

      - USB 3.2 Operation	19

    - PCI Express (PCIe)	19

    - Serial Peripheral Interface (SPI)	21

    - Universal Asynchronous Receiver/Transmitter (UART)	25

    - Controller Area Network (CAN)	26

    - Inter-Chip Communication (I2C)	27

    - Inter-IC Sound (I2S)	29

    - Gigabit Ethernet	30

    - Fan	30

    - Pulse Width Modulator (PWM)	30

Chapter 3.	Power and System Management	32

  - Power Rails	33

  - Power Domains/Islands	33

  - Power Management Controller (PMC)	33

  - Resets	34

  - PMIC_BBAT	34

  - Power Sequencing	34

    - Power Up	34

    - Power Down	35

  - Power States	35

    - ON State	35

    - OFF State	36

    - SLEEP State	36

  - Thermal and Power Monitoring	37

Chapter 4.	Pin Definitions	38

  - Power-on Reset Behavior	38

  - Sleep Behavior	39

  - GPIO	39

  - Pin List	40

Chapter 5.	Electrical, Mechanical, and Thermal Characteristics	44

  - Operating and Absolute Maximum Ratings	44

  - Digital Logic	45

  - Environmental and Mechanical Screening	46

  - Storage and Handling	47

  - Drawing and Dimensions	48

List of Figures

Figure 2-1: SPI Initiator Timing	23

Figure 2-2: SPI Target Timing	24

Figure 3-1: Power State Transition Diagram	35

Figure 5-1: Top View	48

Figure 5-2: Bottom View	49

List of Tables

Table 2-1: GPU Operation	5

Table 2-2: CPU Operation	7

Table 2-3: CSI Pin Descriptions	10

Table 2-4: Camera Pin Descriptions	11

Table 2-5: Supported Video Decode Streams	15

Table 2-6: USB 2.0 Pin Descriptions	18

Table 2-7: USB 3.2 Pin Descriptions	19

Table 2-8: PCIe Pin Descriptions	20

Table 2-9: SPI Mode Descriptions	22

Table 2-10: SPI Pin Descriptions	22

Table 2-11: SPI Initiator Timing Parameters	24

Table 2-12: SPI Target Timing Parameters	24

Table 2-13: UART Pin Descriptions	26

Table 2-14: CAN Pin Descriptions	27

Table 2-15: I2C Pin Descriptions	28

Table 2-16: I2S Pin Descriptions	29

Table 2-17: Gigabit Ethernet Pin Descriptions	30

Table 2-18: PWM Pin Descriptions	31

Table 3-1: Power and System Control Pin Descriptions	32

Table 3-2: PMIC_BBAT Pin Descriptions	34

Table 3-3: OFF State Events	36

Table 3-4: SLEEP and WAKE Events	36

Table 4-1: GPIO Pin Descriptions	39

Table 5-1: Recommended Operating Conditions	44

Table 5-2: Absolute Maximum Ratings	45

Table 5-3: CMOS Pin Type DC Characteristics	45

Table 5-4: Open Drain Pin Type DC Characteristics	46

Table 5-5: Environmental Testing	46

Table 5-6: Typical Handling and Storage Environment	47

Chapter 1 Overview

| Module | Description |
| --- | --- |
| K3-CoM260 32GB | 8-CORE X100™ 64-bit RISC-V CPU + 8-CORE A100™ CPU + 32GB LPDDR5 |
| K3-CoM260 16GB | 8-CORE X100™ 64-bit RISC-V CPU + 8-CORE A100™ CPU + 16GB LPDDR5 |
| Note：References to K3-CoM260 can be read as K3-CoM260 32GB and K3-CoM260 16GB except where explicitly noted. |  |

| Description | Operation/Performance |
| --- | --- |
| AI CPU |  |
| Al-first RISC-V AI-CPU ¦ 8-CORE A100™ 64-bit AI-CPU: - 2 clusters x 4 cores per cluster, each cluster includes 1 MB shared L2 cache and 1.5 MB dedicated accelerator TCM, with cross-cluster access - Each A100 core includes 32 KB I-cache and 32 KB D-cache - Supports RVV 1.0, VLEN: 1024 bits |  |
| Al Compute Performance (up to) | - 60T0PS (MAXN_SUPER) |
| CPU Cluster |  |
| High-performance, 4-issue, out-of-order, multi-core, multi-cluster RISC-V RVA23 processor ¦ 8-CORE X100™ 64-bit RISC-V CPU: - 2 clusters × 4 cores per cluster, each cluster includes 4 MB shared L2 cache, with cross-cluster access - Each X100 core includes 64 KB I-cache and 64 KB D-cache - Compliant with the RVA23 profile - Supports RVV 1.0, VLEN: 256 bits |  |
| Operating Frequency per Core (up to): | - 2.1 GHz (MAXN_SUPER) |
| Memory Subsystem |  |
| Maximum Capacity: 64 GB ¦ Two Channel ¦ 64-bit wide data bus ¦ A Channel Supports Two Rank |  |
| Maximum Operating Frequency (up to) | 2133 MHz ¦ 3200 MHz (MAXN_SUPER) |
| K3-CoM260 32GB Peak Memory Bandwidth: | 64-bit LPDDR5 DRAM 34 GB/s ¦ 51 GB/s (MAXN_SUPER) |
| K3-CoM260 16GB Peak Memory Bandwidth: | 64-bit LPDDR5 DRAM 34 GB/s ¦ 51 GB/s (MAXN_SUPER) |
| HD Video |  |
| Decode Supported Standards: H.265, H.264, VP9 4k60(H2.64/H2.65) |  |
| Encode 1080p60(H2.64/H2.65) |  |
| Display Controller Subsystem |  |
| The display controller with support for eDP/DP and DSI ¦ 1x4K60 DP 1.2 , eDP 1.4 ¦ 1x4K60 MIPI DSI 1.2 |  |
| Maximum Resolution eDP/DP (up to) | 3840x2160 at 60 Hz |
| Maximum Resolution MIPI DSI (up to) | 3840x2160 at 60 Hz |
| PCLK (up to) | 614 MHz |
| Audio Subsystem |  |
| Dedicated programmable audio processor ¦ Arm Cortex A9 with NEON ¦ PDM in/out ¦ Industry-standard High- Definition Audio (HDA) controller provides a multi-channel audio path to the HDMI® interface |  |
| Networking |  |
| 10/100/1000 Gigabit Ethernet ¦ Media Access Controller (MAC) |  |
| Imaging |  |
| Eight lanes MIPI CSI-1.1 ¦ D-PHY 1.1 (12 Gbps) |  |
| Security |  |
| Supports internationally recognized cryptographic algorithms as well as China's commercial cryptography algorithms. - Supports RISC-V PMP and ePMP, and integrates RISC-V IOPMP to deliver high-level security protection. - Supports secure boot, secure storage, and signature verification. - Supports AES, SHA, RSA, SM2, SM3, SM4, and other cryptographic algorithms. - Supports secure management across the entire product lifecycle. |  |
| Storage |  |
| - Supports internal UFS, SD card, and external NVMe - NVMe through PCIe - PCIE0&1, x4 (K3 UPHY0 Lanes [1:0] & UPHY1 Lanes [1:0]), Ctrl #0 - PCIE4, x1 (K3 UPHY4 Lanes [0]), Ctrl #3 |  |
| Peripheral Interfaces |  |
| USB: xHCI host controller with integrated PHY (up to) 2x USB 3.0, 3x USB 2.0 |  |
| PCIe: Up to GEN3 ¦ 3 x1 (or 1 x2 + 1 x1) + 1 x4 ¦ x1 and x2 (supports Root Port only), x4 (supports Root Port or Endpoint modes) |  |
| Audio: 2x I2S/2x Audio Hub (AHUB) ¦ Supports I2S, RJM, LJM, PCM, TDM (multi-slot mode) |  |
| UART: 3 x UART CAN: Single independent CAN port/channel supports connectivity to one CAN network SPI: 2 x SPI I2C: 4 x I2C PWM: 4 x PWM outputs GPIO: 15 x GPIOs |  |
| Mechanical |  |
| Module Size: 69.6 mm x 45 mm ¦ 260 pin SO-DIMM Connector |  |
| Operating Requirements |  |
| Temp. Range (TJ)*: -25°C – 105F°C ¦ Supported Power Input: 5V/5~20V ¦ Operating Lifetime (24x7): 5 years K3-CoM260 32GB Modes: 18W ¦ 25W ¦ 35W (MAXN_SUPER) K3-CoM260 16GB Modes: 18W ¦ 25W ¦ 35W (MAXN_SUPER) |  |
| Notes: - Please refer to the latest K3 Developer Guide (Software Features section) for the most up-to-date list of supported capabilities. Some features may not be available. |  |

Chapter 2. Functional Description

As the world’s first compact edge-computing module based on the RISC-V architecture with full RVA23 support, the SpacemiT K3-CoM260 is ideal for AI edge devices, service robots, and a wide range of autonomous on-device AI agent applications.

The K3-CoM260 32GB module integrates 8x 100™ general-purpose CPU cores and 8x A100™ AI CPU cores, delivering 130 KDMIPS of general compute and 60 TOPS of general AI compute. Real-world testing shows inference performance is greater than 10 Tokens/S@30B. It supports multi-pipeline concurrent AI workloads and high-performance inference tasks. And the included carrier board offers a wide range of I/O interfaces while remaining hardware-compatible with NVIDIA Jetson Orin Nano to simplify hardware migration.

SpacemiT® A100™ RISC-V AI Core

The SpacemiT® A100™ is an Al-first RISC-V AI-CPU that delivers native Al compute capability through the SpacemiT-IME instruction set. Its microarchitecture is specifically optimized for operator-level parallelism, memory bandwidth efficiency, and data locality, enabling highly efficient execution of real-world Al workloads.

In addition to advanced Al acceleration, the A100 fully supports general-purpose CPU functionalities defined by the RVB23 specification and leverages a standard RISC-V unified programming model to power local language models and a broad range of Al-centric applications.

Features:

- Supports 60 TOPs (/Die @FP4 sparse).

- Fully compliant with RISC-V RVB23 standdards.

- Cache Architecture:

  - 32 KB L1 I-Cache and 32 KB L1 D-Cache per core

  - 1 MB L2 Cache per cluster

  - 1.5 MB Scratchpad per cluster

  - L1 D-Cache supports MESI coherence protocol

  - L2 Cache supports MOESI coherence protocol

- Vector Extension: RWV 1.0, VLEN = 1024.

- Advanced Interrupt Architecture (AIA):

  - M-mode MSI: 512

  - S-mode MSI: 512

- Supports ACLINT and APLIC with a total of 512 interrupts.

- Integrated RISC-V Performance Monitoring Unit (PMU).

- Supports SV39 virtual memory management.

- Security Framework:

  - 32 PMP entries compliant with the RISC-V security framework

  - Supports RISC-V Debug and Trace

- RVB23 Optional Extensions (not included in the RVB23 base specification):

  - Vector Crypto: zvkng, zvksg

  - Other Extensions: zvbc, zfh, zbc, zvfh, zfbmin, zvfbfmin, zvfbfwma

  - System and Security: sdtri, svvptc, sspm, smepmp, smstateeen, smcntrpmf

  - SpacemiT® X100™ RISC-V Core

The SpacemiT® X100™ is a high-performance, 4-issue, ouit-of-order, multi-core, multi-cluster RISC-V RVA23 processor optimized for demanding compute scenarios such as servers, autonomous driving systems, and cloud Al inference platforms.

Designed for both performance and robustness, the X100 core provides comprehensive virtualization, strong security resilience, and RAS (Reliability, Availability, Serviceability) capabilities. These characteristics make it a powerful and scalable solution for data-centric and mission-critical applications.

Features:

- Fully compliant with RISC-V RVA23 standards.

- Cache Architecture:

  - 64KB L1 I-Cache and 64KB L1 D-Cache per core

  - 4MB L2 Cache per cluster

  - L1 D-Cache supports MESI coherence protocol

  - L2 Cache supports MOESI coherence protocol

- Vector Extension: RVV 1.0, VLEN = 256.

- Hypervisor Extension: RVH 1.0, GEILEN = 8.

- Advanced Interrupt Architecture (AIA):

  - M-mode MSI: 512

  - S-mode MSI: 512

  - VS-mode MSI: 64

- Supports ACLINT and APLIC with a total of 512 interrupts.

- Supports RISC-V Performance Monitoring Unit (PMU) .

- Supports SV39 virtual memory management.

- Security Framework:

  - 16 PMP entries in accordance with the RISC-V security framework

  - Supports RISC-V Debug, Trace, and RERI frameworks

- RVA23 Optional Extensions (not included in RVA23 base specificaation)：

  - Vector Crypto: zvkng, zvksg

  - Other Extensions: zvbc, zfh, zbc, zvfh, zfbmin, zvfbfmin, zvfbfwma

  - System and Security: sdtri, svvptc, sspm, smepmp, smstateen, smcntrpmf

Table 2-2: CPU Operation

| Module | CPU Cores | CPU Maximum Frequency |
| --- | --- | --- |
| K3-CoM260 16GB | 8-CORE X100™ 64-bit RISC-V CPU + 8-CORE A100™ AI CPU | 2.1 GHz (MAXN_SUPER) |
| K3-CoM260 32GB | 8-CORE X100™ 64-bit RISC-V CPU + 8-CORE A100™ AI CPU | 2.1 GHz (MAXN_SUPER) |

  - Memory Subsystem

2x 16GB 32-bit LPDDR5 DRAM is used on the K3-CoM260 32GB, and 2x 8GB 32-bit LPDDR5 DRAM is used in the K3-CoM260 16GB. It supports the following:

- maximum total capacity of 64 GB.

- Peak Memory Bandwidth: 51GB/s(MAXN_SUPER).

- Maximum operating frequency:  3200 MHz (MAXN_SUPER).

The Dynamic Memory Interface provides a high-performance, low-power connection to external LPDDR4x and LPDDR5 DRAM devices. It supports flexible configurations aand dynamic frequency scaling to balance bandwidth and power efficiency across various application scenarios.

Features:

- Supports LPDDR4x and LPDDR5 memory types:

  - LPDDR4x data rate up to 4266 MT/s

  - LPDDR5 data rate up to 6400 MT/s

- Supports two channels, each with a 32-bit data width.

- Each channel supports two ranks.

- Supports dynamic frequency scaling, allowing real-time ffrequency adjustment based on bandwidth demand to optimize power efficiency.

  - Memory

The K3-CoM260 32GB integrates 32GB 64-bit LPDDR5 DRAM, and K3-CoM260 16GB integrates 16GB 64-bit LPDDR5 DRAM. Maximum frequency of Jetson Orin Nano Memory is 2133 MHz or 3200MHz (MAXN_SUPER). The theoretical peak memory bandwidth on K3-CoM260 32GB is 34 GB/s or 51 GB/s (MAXN_SUPER), and on K3-CoM260 16GB is 34 GB/s or 51 GB/s (MAXN_SUPER).

  - Video Input Interfaces

    - MIPI Camera Serial Interface (CSI)

Standard

MIPI CSI 1.1 Receiver specification

MIPI D-PHY® 1.1 Physical Layer specification

The K3-CoM260 interface integrates three MIPI-CSI2 v1.1 controllers, each equipped with four data lanes supporting a maximum transfer rate of 1.5 Gbps per lane.

Features:

- Double-sensor:

  - 4-Lane + 4-Lane

  - 4-Lane + 2-Lane

- Triple-sensor:

  - 2-Lane + 2-Lane+4-Lane

  - 2-Lane + 2-Lane+2-Lane

- Quad-sensor:

  - 2-Lane + 2-Lane + 2-Lane + 2-Lane

- Supported input data formats:

  - Legacy YUV420 8-bit

  - YUV420 8-bit

  - RAW8

  - RAW10

  - RAW12

  - Embedded data type

- Supported data interleaving types:

  - Data-type interleaving

  - Virtual-channel interleaving

Table 2-3: CSI Pin Descriptions

| Pin # | Signal Name | Description | Direction | Pin Type |
| --- | --- | --- | --- | --- |
| 10 | MIPI_CSI0_CLKN | Camera, CSI 0 Clock– | Input | MIPI D-PHY |
| 12 | MIPI_CSI0_CLKP | Camera, CSI 0 Clock+ | Input | MIPI D-PHY |
| 4 | MIPI_CSI0_DN0 | Camera, CSI 0 Data 0– | Input | MIPI D-PHY |
| 6 | MIPI_CSI0_DP0 | Camera, CSI 0 Data 0+ | Input | MIPI D-PHY |
| 16 | MIPI_CSI0_DN1 | Camera, CSI 0 Data 1– | Input | MIPI D-PHY |
| 18 | MIPI_CSI0_DP1 | Camera, CSI 0 Data 1+ | Input | MIPI D-PHY |
| 9 | MIPI_CSI1_CLKN | Camera, CSI 1 Clock– | Input | MIPI D-PHY |
| 11 | MIPI_CSI1_CLKP | Camera, CSI 1 Clock+ | Input | MIPI D-PHY |
| 3 | MIPI_CSI0_DN2 (MIPI_CSI1_DN0) | Camera, CSI 0 Data 2– (Camera, CSI 1 Data 0–) | Input | MIPI D-PHY |
| 5 | MIPI_CSI0_DP2 (MIPI_CSI1_DP0) | Camera, CSI 0 Data 2+ (Camera, CSI 1 Data 0+) | Input | MIPI D-PHY |
| 15 | MIPI_CSI0_DN3 (MIPI_CSI1_DN1) | Camera, CSI 0 Data 3– (Camera, CSI 1 Data 1–) | Input | MIPI D-PHY |
| 17 | MIPI_CSI0_DP3 (MIPI_CSI1_DP1) | Camera, CSI 0 Data 3+ (Camera, CSI 1 Data 1+) | Input | MIPI D-PHY |
| 28 | MIPI_CSI2_CLKN | Camera, CSI 2 Clock– | Input | MIPI D-PHY |
| 30 | MIPI_CSI2_CLKP | Camera, CSI 2 Clock+ | Input | MIPI D-PHY |
| 22 | MIPI_CSI2_DN0 | Camera, CSI 2 Data 0– | Input | MIPI D-PHY |
| 24 | MIPI_CSI2_DP0 | Camera, CSI 2 Data 0+ | Input | MIPI D-PHY |
| 34 | MIPI_CSI2_DN1 | Camera, CSI 2 Data 1– | Input | MIPI D-PHY |
| 36 | MIPI_CSI2_DP1 | Camera, CSI 2 Data 1+ | Input | MIPI D-PHY |
| 27 | MIPI_CSI3_CLKN | Camera, CSI 3 Clock– | Input | MIPI D-PHY |
| 29 | MIPI_CSI3_CLKP | Camera, CSI 3 Clock+ | Input | MIPI D-PHY |
| 21 | MIPI_CSI2_DN2 | Camera, CSI 2 Data 2– | Input | MIPI D-PHY |
| 23 | MIPI_CSI2_DP2 | Camera, CSI 2 Data 2+ | Input | MIPI D-PHY |
| 33 | MIPI_CSI2_DN3 | Camera, CSI 2 Data 3– | Input | MIPI D-PHY |
| 35 | MIPI_CSI2_DP3 | Camera, CSI 2 Data 3+ | Input | MIPI D-PHY |

Table 2-4: Camera Pin Descriptions

| Pin # | Signal Name | Description | Direction | Pin Type |
| --- | --- | --- | --- | --- |
| 116 | CLK_CAMCK1 | CSI 0 Reference Clock | Output | CMOS – 1.8V |
| 114 | MIPI_CSI0_PWDN | CSI 0 Powerdown or GPIO | Output | CMOS – 1.8V |
| 122 | MIPI_CSI2_PWDN | CSI 2 Reference Clock | Output | CMOS – 1.8V |
| 120 | CLK_CAMCK3 | CSI 2 Powerdown or GPIO | Output | CMOS – 1.8V |
| 88 | (RSVD)CLK_CAMCK2 | CSI 1 Reference Clock | Output | CMOS – 1.8V |
| 94 | (RSVD)MIPI_CSI1_PWDN | CSI 1Powerdown or GPIO | Output | CMOS – 1.8V |
| 92 | (RSVD)MIPI_CSI3_PWDN | CSI 3 Reference Clock | Output | CMOS – 1.8V |
| 90 | (RSVD)CLK_CAMCK4 | CSI 3 Powerdown or GPIO | Output | CMOS – 1.8V |

  - Sensor Processing Engine

  - Security Subsystem

This subsystem is comprised of the following:

- Crypto Engine

- TRNG

- eFUSE

- IOPMP

    - Crypto Engine

Supports internationally recognized cryptographic algorithms as vvell as China's commercial cryptography algorithms.

Features:

- Hash Algorithms: SHA1/224/256, SM3

- Symmetric Algorithms: AES128/192/256, SM4

- Asymmetric Algorithms: RSA1024/2048/4096, ECC128/256/512, SM2

    - TRNG

A random number generator compliant with China's commercial cryptography standards.

Features:

- Built-in 32-bit TRNG.

- Ensures randomness, unpredictability, and non-reproducibility.

    - eFUSE

Integrated 4096-bit eFuse, divided into 16 banks of 256 bits each, with 256 bits available for user

customization.

Features:

- Supports eFuse bank locking.

- Supports automatic hardware parameter loading.

- Supports lifecycle management.

- Supports secure boot configuration.

- Supports storage of root keys and encryption-protected keys.

- Supports 256-bit non-volatile counter (NV Counter).

    - IOPMP

The IOPMP (I/O Physical Memory Protection) module is designed in coordination with the PMP to ensure secure access control across the platform's peripherals. While the PMP validates bus accesses initiated by the RISC-V cores,the IOPMP verifies transactions issued by other bus masters or subsystems. Configured exclusively by the secure world, the IOPMP defines access permissions and attributes for transactions initiated by non-CPU masters. All transactions are subject to IOPMP entry checks, and access granted only when the permission verification passes.

Features:

- Supports access control for read, write, and execute permissions.

- Bus requests incur a one-cycle delay after permission checking.

- Supports logging of access violation information.

- Supports interrupt generation for access violation events.

- Integrates 9 IOPMPs to provide security control for hardware modules and subsystems.

  - Display Controller

The SpacemiT K3-CoM260 provides 1x  MIPI DSI and DP port.The Display Controller is a hardware module that transfers display data from the internal memory to the DSI and DP/eDP controllers. It supports high-resolution panels and advanced image processing features.

Supported Display features are:

- Up to HD+:

  - 3840 x 2160 @ 60fps

  - 2560 x 1440 @ 144fps

- Layer Composition:

  - Up to 4 full-size layer composers

  - Maximum 16 layer composers via up-down layer reuse in the RIMA channel

- Command & Write-Back:

  - cmdlist mechanism to configure hardware registers

  - Concurrent write-back for raw and AFBC formats

  - Dithering, cropping, and rotation supported in write-back path

- Memory Management:

  - Advanced MMU with nearly no page misses during 90° and 270° rotation

- Color & Display Enhancements:

  - Color keying and solid color generation

  - Advanced error diffusion and pattern-based dithering

  - Color saturation and contrast enhancement

  - Display effect adjustment

- Input Formats:

  - ABGR2101010, ARGB2101010, BGRA210101010, RGBA2101010

  - ABGR8888, ARGB8888, BGRA8888, RGBA8888

  - XBGR8888, XRGB8888, BGRX8888, RGBX8888

  - BGR888, RGB888, ABGR1555, RGBA5551, BGR565/RGB565

  - XYUV_444_P1_8, XYUV_444_P1_10, YVYU_422_P1_8, VYUY_422_P1_8

  - YUV_420_P2_8, YUV_420_P3_8

- Output Formats:

  - RGB888, RGB565, RGB666

- Panel & Mode Support:

  - Video mode and command mode (frame buffer in LCM)

  - Dynamic DDR frequency adjustment with embedded DFC buffer

- Source Format Support:

  - Both AFBC and raw image sources

The MIPI Display Serial Interface (MIPI DSI) is a high-speed interface connecting the host processor to display peripherals, fully compliant with MIPI Alliance specifications.The DP Controller is a display interface controller that manages data transfer from the SoC to external DisplayPort (DP) panels.

DP is a digital display interface often used to connect a video source to a display device over a cable, in consumer or commercial applications.

Supported DisplayPort features are:

- Compliance with DisplayPort (DP) standard v1.2.

- Compliance with embedded DisplayPort (eDP) standardv1.4

- Supports resolutions up to 3840 x 2160 @ 60fps or 2560 x 1440 @ 144fps

- The MIPI Display Serial Interface (MIPI DSI) is a high-speed interface connecting the host processor to display peripherals, fully compliant with MIPI Alliance specifications formobile and embedded devices.

Supported DisplayPort features are:

- Standards Compliance:

  - MIPI DSI v1.2

  - MIPI D-PHY v1.2

  - Display Command Set (DCS) standard

- Lane & Speed Support:

  - Up to 4 data lanes

  - Maximum speed up to 4.5 Gbps per lane

  - 1 active panel per D-PHY link

- Resolution Support:

  - Up to 3840 x 2160 @ 60fps or 2560 x 1440 @ 90fps

- Operational Modes:

  - Command mode, video mode, and video burst mode

- Signaling Support:

  - HS-TX (High-Speed Transmit)

  - LP-TX / LP-RX (Low-Power Transmit / Receive)

  - LP-CLK / LP-CD (Low-Power Clock / Data)

- Data & Channel Support:

  - Support for all pixel formats defined in DSI and DCS

  - Support for virtual channels in the MIPI link

  - Burst video mode support with D-PHY up to 4.5 Gbps per lane

  - Audio Subsystem

The K3-CoM260 integrates a comprehensive Audio Subsystem designed to deliver high-quality, low-latency audio performance. It incorporates multiple PS and DisplayPort audio interfaces to support diverse playback and recording scenarios across multimedia and communication applications.

The subsystem includes the following primary interfaces:

- 2 x Full-Duplex l2S Interfaces

- 1 x Half-Duplex l2S Interfaces (connected to the DP/eDP controller)

- 1 x DP/eDP Audio Interfaces

  - Video Subsystem

- The Video Processing Unit (VPU) is quad-core video accelerator designed to handle both decoding and encoding of multiple video standards. It includes a host CPU that runs firmware to control the hardware engine, managing tasks such as bitstream parsing, sub-block control, and error resilience.

- The VPU can operate at up to 1.2 GHz and supports a wide range of video standards, including H.265, H.264, VP8, VP9, MPEG4, MPEG2, and H.263. It allows simultaneous operations such as:

- Encoding and decoding at 1080p60.

- H.264/H.265 encoding at 1080p60 and H.264/H.265 decoding at 4K60.

- The video codec core performs the actual decoding and encoding for each standard using dedicated hardware logic. The Macroblock Sequencer serves as the main controller, scheduling the process flows of sub-blocks to reduce the load on the processor and simplify firmware complexity.

- Additionally, several standard-independent blocks share common logic during operation, ensuring high efficiency and streamlined performance across different video standards.

    - Video Decoder

Video decode standards supported: H.265 (HEVC), H.264, VP9, VP8, VC-1, MPEG-4, MPEG-2, and H2.63.

Table 2-5: Supported Video Decode Streams

| Standard | Profile(s) | Resolution (Maximum Number of Streams) | Throughput MPix/s (Up to) | Maximum Cumulative Bitrate (Mbps) |
| --- | --- | --- | --- | --- |
| H.264 | Baseline, Main, High Profiles | 4K120 (1) ¦ 4K60 (2) ¦ 1080p60 (8) ¦ 1080p30 (16) | 450 | 115 |
|  | High 444, High 444 Predictive, MVC (per view considering two views) (see note) | 1x1080p60 ¦ 3x 1080p30 | 225 | 100 |
| H.265 (HEVC) | Main, Main10 | 4K60 (1) ¦ 4K30 (2) ¦ 1080p60 (5) ¦ 1080p30 (11) | 690 | 150 |
|  | Main 444, Main 444 10, MV (per view) | 4K30 (1) ¦ 1080p60 (2) ¦ 1080p30 (5) | 340 | 100 |
| VP9 | Profile 0, Profile 2 | 4K60 (1) ¦ 4K30 (2) ¦ 1080p60 (5) ¦ 1080p30 (10) | 625 | 100 |

  - Interface Descriptions

The following sections outline the interfaces available on the K3-CoM260 module and details the module pins used to interact with and control each interface.

    - Universal Serial Bus (USB)

| Standard | Notes |
| --- | --- |
| Universal Serial Bus Specification Revision 3.2 Gen1 and Gen2 |  |
| Universal Serial Bus Specification Revision 2.0 | - Modes: Host and Device (Only USB 2.0 port USB0 supports RCM, Host, Device Mode. All other ports are Host only) - Speeds: Low, Full, and High - USB Battery Charging 1.2 Specifications |
| Enhanced Host Controller Interface Specification for Universal Serial Bus revision 1.0 | - |

The K3-CoM260 integrates multiple USB interfaces to support high-speed connectivity and flexible device configurations. The USB subsystem includes the following ports:

- USB 2.0 Host Port

- USB 3.0 Dual-Role Device(DRD) Port an integrated USB2.0 DRD interface (USB3.0 Port A)

- Three USB3.0 Host Ports (USB3.0 Port B/C/D) - their SuperSpeed PHYs are shared with PCIe and can operate in either USB or PCIe mode, but only one function can be selected at a time

The registers and data structures of the USB 2.0 and USB 3.0 controllers comply with the Intel xHCI specification. Supports USB 2.0 suspend mode and USB 3.0 U1/U2/U3 low-power states.

      - USB 2.0 Operation

Supported Display features are:

- Controller

  - Compliant with the USB 2.0 specification

  - Only USB Port A supports both Host and Device mode

  - Supports High-Speed (480 Mb/s), Full-Speed (12 Mb/s), andLow-Speed (1.5 Mb/s) operation

- Communication Interface

  - Utilizes a UTMI+ (30/60 MHZ) interface for USB2.0 PHY

- Clock Domains

  - UTMI+ PHY (30/60 MHz)

  - MAC (125 MHz nominal)

  - Bus clock domain

  - RAM clock domain

- System & Power Management

  - Integrated DMA controller

  - Supports USB2.0 suspend mode

- Endpoints & Memory

  - Supports up to 32 endpoints in Device mode

  - Flexible endpoint FIFO sizing (not restricted to powers of 2) for contiguous memory allocation

  - Supports descriptor caching and data prefetching to improve pperformance in high-latency systems

- Additional Features

  - Software-controlled USB standard commands (SETUP packets can be forwarded to the application for decoding)

  - Hardware-level error handling for USB bus and packet-level errors

  - Interrupt support

Table 2-6: USB 2.0 Pin Descriptions

| Pin # | Signal Name | Description | Direction | Pin Type |
| --- | --- | --- | --- | --- |
| 109 | USB20_A_DRD_USB_M | USB 2.0 DRD Port A Data– | Bidir | USB PHY |
| 111 | USB20_A_DRD_USB_P | USB 2.0 DRD Port A Data+ | Bidir | USB PHY |
| 115 | USB20_B_USB_M | USB 2.0 Port B Data– | Bidir | USB PHY |
| 117 | USB20_B_USB_P | USB 2.0 Port B Data+ | Bidir | USB PHY |
| 121 | USB20_HOST_M | USB 2.0 Host Data– | Bidir | USB PHY |
| 123 | USB20_HOST_P | USB 2.0 Host Data+ | Bidir | USB PHY |

      - USB 3.0 Operation

USB3.0 PORT A supports both host and device modes, while others only support host mode. All USB3.0 ports support a maximum data transfer rate of 5 Gb/s.

Note: The USB3.0 Port A SuperSpeed PHY integrates an internal Type-Corientation switch controllable via GPIO input.

Table 2-7: USB 3.2 Pin Descriptions

| Pin # | Signal Name | Description | Direction | Pin Type |
| --- | --- | --- | --- | --- |
| 161 | PCIe2/USB3-B_RX0n | USB SS Receive– (USB 3.0 Host Ctrl #1) | Input | USB SS PHY |
| 163 | PCIe2/USB3-B_RX0p | USB SS Receive+ (USB 3.0 Host Ctrl #1) | Input | USB SS PHY |
| 166 | PCIe2/USB3-B_TX0n | USB SS Transmit– (USB 3.0 Host Ctrl #1) | Output | USB SS PHY |
| 168 | PCIe2/USB3-B_TX0p | USB SS Transmit+ (USB 3.0 Host Ctrl #1) | Output | USB SS PHY |
| 39 | USB_DRD_RX1N | USB SS Receive– (USB 3.0 OTG Ctrl #0) | Input | USB SS PHY |
| 41 | USB_DRD_RX1P | USB SS Receive+ (USB 3.0 OTG Ctrl #0) | Input | USB SS PHY |
| 45 | USB_DRD_TX1N | USB SS Transmit– (USB 3.0 OTG Ctrl #0) | Output | USB SS PHY |
| 47 | USB_DRD_TX1P | USB SS Transmit+ (USB 3.0 OTG Ctrl #0) | Output | USB SS PHY |
| 51 | USB_DRD_RX2N | USB SS Receive– (USB 3.0 OTG Ctrl #0) | Input | USB SS PHY |
| 53 | USB_DRD_RX2P | USB SS Receive+ (USB 3.0 OTG Ctrl #0) | Input | USB SS PHY |
| 57 | USB_DRD_TX2N | USB SS Transmit– (USB 3.0 OTG Ctrl #0) | Output | USB SS PHY |
| 59 | USB_DRD_TX2P | USB SS Transmit+ (USB 3.0 OTG Ctrl #0) | Output | USB SS PHY |

    - PCI Express (PCIe)

| Standard | Notes |
| --- | --- |
| PCI Express Base Specification Revision 3.0 |  |

The K3-CoM260 module integrates four PCIe controllers supporting:

- Connections to three interfaces, 2 x1  + 1 x4 + 1 x1GEN3.

- x1 (supports Root Port only) , x4 (supports Root Port or Endpoint modes) upstream and downstream AXI interfaces that serve as the control path from the K3-CoM260 to the external PCIe device.

- Gen4 (8 GT/s) supported on all controllers/lanes

- Four PCIe controllers, seven lanes for a total of 56GT/s.

- Controller #0 can operate in x1, x2, or x4 mode.

- Controller #2 can operate in x2 mode if USB3.0 Host Controller #1 is not used . In these cases, Controller #2 can operate in x1 mode.

- Controller #3 can operate in x1 mode.

- Controller #4 can operate in x1 mode.

Table 2-8: PCIe Pin Descriptions

| Pin # | Signal Name | Description | Direction | Pin Type |
| --- | --- | --- | --- | --- |
| 131 | PCIe0_RX0n | PCIe 0 Receive 0– (PCIe Ctrl #0 Lane 0) | Input | PCIe PHY |
| 133 | PCIe0_RX0p | PCIe 0 Receive 0+ (PCIe Ctrl #0 Lane 0) | Input | PCIe PHY |
| 137 | PCIe0_RX1n | PCIe 0 Receive 1– (PCIe Ctrl #0 Lane 1) | Input | PCIe PHY |
| 139 | PCIe0_RX1p | PCIe 0 Receive 1+ (PCIe Ctrl #0 Lane 1) | Input | PCIe PHY |
| 149 | PCIe1_RX0n | PCIe 1 Receive 0– (PCIe Ctrl #0 Lane 2) | Input | PCIe PHY |
| 151 | PCIe1_RX0p | PCIe 1 Receive 0+ (PCIe Ctrl #0 Lane 2) | Input | PCIe PHY |
| 155 | PCIe1_RX1n | PCIe 1 Receive 1– (PCIe Ctrl #0 Lane 3) | Input | PCIe PHY |
| 157 | PCIe1_RX1p | PCIe 1 Receive 1+ (PCIe Ctrl #0 Lane 3) | Input | PCIe PHY |
| 134 | PCIe0_TX0n | PCIe 0 Transmit 0– (PCIe Ctrl #0 Lane 0) | Output | PCIe PHY |
| 136 | PCIe0_TX0p | PCIe 0 Transmit 0+ (PCIe Ctrl #0 Lane 0) | Output | PCIe PHY |
| 140 | PCIe0_TX1n | PCIe 0 Transmit 1– PCIe Ctrl #0 Lane 1) | Output | PCIe PHY |
| 142 | PCIe0_TX1p | PCIe 0 Transmit 1+ (PCIe Ctrl #0 Lane 1) | Output | PCIe PHY |
| 148 | PCIe1_TX0n | PCIe 1 Transmit 0– (PCIe Ctrl #0 Lane 2) | Output | PCIe PHY |
| 150 | PCIe1_TX0p | PCIe 1 Transmit 0+ (PCIe Ctrl #0 Lane 2) | Output | PCIe PHY |
| 154 | PCIe1_TX1n | PCIe 1 Transmit 1– (PCIe Ctrl #0 Lane 3) | Output | PCIe PHY |
| 156 | PCIe1_TX1p | PCIe 1 Transmit 1+ (PCIe Ctrl #0 Lane 3) | Output | PCIe PHY |
| 181 | PCIeA_PERSTn | PCIe 0 Reset (PCIe Ctrl #0). 4.7kΩ pull-up to 3.3V on the module. Output when K3-CoM260 is Root Port or input when K3-CoM260 is Endpoint. | Bidir | Open Drain 3.3V |
| 180 | PCIeA_CLKREQn | PCIE 0 Clock Request (PCIe Ctrl #0). 47kΩ pull-up to 3.3V on the module. Input when K3-CoM260 is Root Port or output when K3-CoM260 is Endpoint. | Bidir | Open Drain 3.3V |
| 160 | PCIe0_REFCLK_n | PCIe #0 Reference Clock- (reference clock input when K3-CoM260 is an Endpoint). | Bidir | PCIe PHY |
| 162 | PCIe0_REFCLK_P | PCIe #0 Reference Clock+ (reference clock input when K3-CoM260 is an Endpoint). | Bidir | PCIe PHY |
| 167 | PCIe5_RX0n | PCIe 5 Receive 0– (PCIe Ctrl #4 Lane 0) | Input | PCIe PHY |
| 169 | PCIe5_RX0p | PCIe 5 Receive 0+ (PCIe Ctrl #4 Lane 0) | Input | PCIe PHY |
| 172 | PCIe5_TX0n | PCIe 5 Transmit 0– (PCIe Ctrl #4 Lane 0) | Output | PCIe PHY |
| 174 | PCIe5_TX0p | PCIe 5 Transmit 0+ (PCIe Ctrl #4 Lane 0) | Output | PCIe PHY |
| 183 | PCIeE_PERSTn | PCIe 5 Reset (PCIe Ctrl #4). 4.7kΩ pull-up to 3.3V on the module. | Output | Open Drain 3.3V |
| 182 | PCIeE_CLKREQn | PCIe 5 Clock Request (PCIe Ctrl #4). 47kΩ pull-up to 3.3V on the module. | Bidir | Open Drain 3.3V |
| 173 | PCIe5_REFCLK_n | PCIe 5 Reference Clock– (PCIe Ctrl #4) | Output | PCIe PHY |
| 175 | PCIe5_REFCLK_p | PCIe 5 Reference Clock+ (PCIe Ctrl #4) | Output | PCIe PHY |
| 179 | PCIeA_WAKEn | PCIe Wake. 47kΩ pull-up to 3.3V on the module. | Bidir | Open Drain 3.3V |
| 40 | PCIe4/USB3-D_RX0n (PCIe2/USB3-B_RX0n) | PCIe 4 Receive 0– (PCIe Ctrl #3 Lane 0) or PCIe 2 Receive 0– (PCIe Ctrl #2 Lane 0) | Input | PCIe PHY |
| 42 | PCIe4/USB3-D_RX0p (PCIe2/USB3-B_RX0p) | PCIe 4 Receive 0+ (PCIe Ctrl #3 Lane 0) or PCIe 2 Receive 0+ (PCIe Ctrl #2 Lane 0) | Input | PCIe PHY |
| 46 | PCIe4/USB3-D_TX0n (PCIe2/USB3-B_TX0n) | PCIe 4 Transmit 0– (PCIe Ctrl #3 Lane 0) or PCIe 2 Receive 0– (PCIe Ctrl #2 Lane 0) | Output | PCIe PHY |
| 48 | PCIe4/USB3-D_TX0p (PCIe2/USB3-B_TX0p) | PCIe 4 Transmit 0+ (PCIe Ctrl #3 Lane 0) or PCIe 2 Receive 0+ (PCIe Ctrl #2 Lane 0) | Output | PCIe PHY |
| 58 | PCIe3/USB3-C_RX0n | PCIe 3 Receive 1– (PCIe Ctrl #2 Lane 1) | Input | PCIe PHY |
| 60 | PCIe3/USB3-C_RX0p | PCIe 3 Receive 1+ (PCIe Ctrl #2 Lane 1) | Input | PCIe PHY |
| 64 | PCIe3/USB3-C_TX0n | PCIe 3 Transmit 1– (PCIe Ctrl #2 Lane 1) | Output | PCIe PHY |
| 66 | PCIe3/USB3-C_TX0p | PCIe 3 Transmit 1+ (PCIe Ctrl #2 Lane 1) | Output | PCIe PHY |
| 52 | PCIe4_REFCLK_n (PCIe2_REFCLK_n) | PCIe 4 Reference Clock– (PCIe Ctrl #3) or PCIe 2 Reference Clock– (PCIe Ctrl #2) | Output | PCIe PHY |
| 54 | PCIe4_REFCLK_p (PCIe2_REFCLK_n) | PCIe 4 Reference Clock+ (PCIe Ctrl #3) or PCIe 2 Reference Clock– (PCIe Ctrl #2) | Output | PCIe PHY |
| 219 | PCIeD_PERSTn | PCIe 4 Reset (PCIe Ctrl #3). 4.7kΩ pull-up to 3.3V on the module. | Output | Open Drain 3.3V |
| 221 | PCIeD_CLKREQn | PCIe 4 Clock Request (PCIe Ctrl #3). 47kΩ pull-up to 3.3V on the module. | Bidir | Open Drain 3.3V |
| 229 | PCIe3_REFCLK_p | PCIe 3 Reference Clock– (PCIe Ctrl #2) | Output | PCIe PHY |
| 227 | PCIe3_REFCLK_n | PCIe 3 Reference Clock+ (PCIe Ctrl #2) | Output | PCIe PHY |
| 223 | PCIeC_PERSTn | PCIe 3 Reset (PCIe Ctrl #2). 4.7kΩ pull-up to 3.3V on the module. | Output | Open Drain 3.3V |
| 225 | PCIeC_CLKREQn | PCIe 3 Clock Request (PCIe Ctrl #2). 47kΩ pull-up to 3.3V on the module. | Bidir | Open Drain 3.3V |

See the K3 Datasheet for supported USB 3.0/PCIe configuration and connection examples.

    - Serial Peripheral Interface (SPI)

The Serial Peripheral Interface (SPI) controller allows a duplex, synchronous, serial communication between the controller and external peripheral devices; it supports both Master and Slave modes of operation on the SPI bus. See the K3 Datasheet for more information.

Features:

- 2x SPI Interface.

- Supports all four CPOL/CPHA combinations defined by the SPI specification.

- Configurable for operation in either Master or Slave mode.

- Supports receive-without-transmit operation.

- Supports serial bit rates: 812.5K, 1.625M, 3.25M, 6.5M, 13M, 26M and 52Mbps.

- Data size configurable to 8, 16, 24, or 32 bits.

- Equipped with independent transmit (TXFIFO) and receive(RXFIFO) buffers:

  - In Non-Packed Data Mode, both FIFOs are 32 entries x 32 bits, supporting a total of 32 samples.

  - In Packed Data Mode, double-depth FIFOs are used for 8-bit or 16-bit data, providing 64 entries x 16 bits, supporting a total of 64 samples.

  - Both FIFOs support loading and unloading via Programmed I/O (PIO) Or DMA burst transfers.

Table 2-9: SPI Mode Descriptions

| SPI Mode | Clock Polarity | Clock Phase | SCK Inactive State | Data Latch In | Data Latch Out |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | Low | Latched IN on the positive edge of clock | Latched OUT on the negative edge of clock |
| 1 | 0 | 1 | Low | Latched IN on the negative edge of clock | Latched OUT on the positive edge of clock |
| 2 | 1 | 0 | High | Latched IN on the negative edge of clock | Latched OUT on the positive edge of clock |
| 3 | 1 | 1 | High | Latched IN on the positive edge of clock | Latched OUT on the negative edge of clock |

Table 2-10: SPI Pin Descriptions

| Pin # | Signal Name | Description | Direction | Pin Type |
| --- | --- | --- | --- | --- |
| 91 | SPI0_SCK | SPI 0 Clock | Bidir | CMOS – 1.8V |
| 89 | SPI0_MOSI | SPI 0 Master Out / Slave In | Bidir | CMOS – 1.8V |
| 93 | SPI0_MISO | SPI 0 Master In / Slave Out | Bidir | CMOS – 1.8V |
| 95 | SPI0_CS | SPI 0 Chip Select 0 | Bidir | CMOS – 1.8V |
| 97 | SPI0_CS1 | SPI 0 Chip Select 1 | Bidir | CMOS – 1.8V |
| 106 | R-SPI0_SCK | RT24 CPU SPI 0 Clock | Bidir | CMOS – 1.8V |
| 104 | R-SPI0_MOSI | RT24 CPU SPI 0 Master Out / Slave In | Bidir | CMOS – 1.8V |
| 108 | R-SPI0_MISO | RT24 CPU SPI 0 Master In / Slave Out | Bidir | CMOS – 1.8V |
| 110 | R-SPI0_CS | RT24 CPU SPI 0 Chip Select 0 | Bidir | CMOS – 1.8V |
| 112 | R-SPI0_CS1 | RT24 CPU SPI 0 Chip Select 1 | Bidir | CMOS – 1.8V |

Figure 2-1: SPI Initiator Timing

tCS

tCH

tCL

tCSS

tCSH

n

tCSS	tCSH

tSU

0

tHD

MSB IN

LSB IN

tDD

MSB OUT

LSB OUT

SPIx_CSx_N

SPIx_SCK

SPIx_MISO

SPIx_MOSI

Table 2-11: SPI Initiator Timing Parameters

| Symbol | Parameter | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| Fsck | SCK Clock Frequency |  | 81.6 | MHz |
| Fsck | SCK Clock Frequency (FSI SPI) |  | 40 | MHz |
| tSCP | SCK Period | 1000 * 1/Fsck(max) | 1000 * 1/Fsck(min) | ns |
| tCH | SCK high time | 45% * tSCP | 55% * tSCP | ns |
| tCL | SCK low time | 45% * tSCP | 55% * tSCP | ns |
| tCRT | SCK Rise time (slew rate) | 0.1 |  | V/ns |
| tCFT | SCK Fall time (slew rate) | 0.1 |  | V/ns |
| tSU | Data setup time (MISO) | 2 |  | ns |
| tHD | Data hold time (MISO) | 3 |  | ns |
| tDD | Active Clock edge to MOSI data Output Valid |  | 6 | ns |
| tCSS | CSx_N setup time | 2 |  | ns |
| tCSH | CSx_N hold time | 3 |  | ns |
| tCS | CSx_N high time | 10 |  | ns |

Figure 2-2: SPI Target Timing

tCS

tCSS

tSCP

tCH

1

tCL

tCSH

n

tDSU

0

tDH

tDD

SPIx_CSx_N

SPIx_SCK

SPIx_MOSI

SPIx_MISO

Table 2-12: SPI Target Timing Parameters

| Symbol | Parameter | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| Fsck | SCK Clock Frequency |  | 51 | MHz |
| Fsck | SCK Clock Frequency (FSI SPI) |  | 26 | MHz |
| tSCP | SCK Period | 20 |  | ns |
| tCH | SCK high time | 45% * tSCP | 55% * tSCP | ns |
| tCL | SCK low time | 45% * tSCP | 55% * tSCP | Ns |
| tCSS | CSx_N setup time | 1 * tSCP |  | ns |
| tCSH | CSx_N hold time | 1 * tSCP |  | ns |
| tCS | CSx_N high time | 1 * tSCP |  | ns |
| tSU | Data setup time (MOSI) | 4 |  | ns |
| tHD | Data hold time (MOSI) | 2 |  | ns |
| tDD | Active Clock edge to MISO data Output Valid | 2.5 | 17 | ns |
| tHO | MISO Output Hold Time | 2 |  | ns |

    - Universal Asynchronous Receiver/Transmitter (UART)

The UART (Universal Asynchronous Receiver/Transmitterr) module provides asynchronous serial communication between the system and external devices. It supports flexible configuration, efficient data handling, and diagnostic features, suitable for both low- and high-speed communication scenarios.

Features:

- 3x UART Interface.

- Fully compatible with industry-standard 8250 UART.

- Automatic insertion and removal of start, stop, and parity bits in the serial data stream.

- Independent control of transmit, receive, line status, and data set interrupts.

- Auto Flow Control:

  - RTS (output): automatically driven by UART receive FIFO

  - CTS (input): controlled by external modem transmission signal

- Programmable Serial Parameters:

  - Character length: 7 or 8 bits

  - Parity: even, odd, or none

  - Stop bits: 1

  - Baud rate: up to 3.6 Mbps for 4 high-speed UARTS.

  - False start-bit detection supported

- FIFO Buffers:

  - 256-byte transmit FIFO

  - 256-byte receive FIFO

- Diagnostics:

  - Loopback mode for communication link verification

  - Break, parity, and framing error simulation

- Independent DMA request channels for transmit and receive operations.

Table 2-13: UART Pin Descriptions

| Pin # | Signal Name | Description | Direction | Pin Type |
| --- | --- | --- | --- | --- |
| 99 | UART6_TXD | UART #6 Transmit | Output | CMOS – 1.8V |
| 101 | UART6_RXD | UART #6 Receive | Input | CMOS – 1.8V |
| 103 | UART6_RTS | UART #6 Request to Send | Output | CMOS – 1.8V |
| 105 | UART6_CTS | UART #6 Clear to Send | Input | CMOS – 1.8V |
| 203 | UART1_TXD | UART #1 Transmit | Output | CMOS – 1.8V |
| 205 | UART1_RXD | UART #1 Receive | Input | CMOS – 1.8V |
| 207 | UART1_RTS | UART #1 Request to Send | Output | CMOS – 1.8V |
| 209 | UART1_CTS | UART #1 Clear to Send | Input | CMOS – 1.8V |
| 236 | UART0_TXD_PMU | UART #0 Transmit | Output | CMOS – 1.8V |
| 238 | UART0_RXD_PMU | UART #0 Receive | Input | CMOS – 1.8V |

    - Controller Area Network (CAN)

| Standard | Notes |
| --- | --- |
| ISO 16845-1:2016 | CAN conformance test |
| ISO 11898-1 | CAN standard |

The K3-CoM260 integrates CAN-FD interface. CAN-FD controller is a full implementation of the CAN protocol, compliant with both CAN with Flexible Data-Rate (CAN-FD) and CAN 2.0 Part B specifications, enabling high-performance automotive and industrial communication.

Features:

- Full compliance with CAN-FD protocol and CAN 2.0 Part B, supporting:

  - Standard and extended data frames

  - Data payloads from 0 to 64 bytes

  - Programmable bit rates

  - Content-related addressing

- configurable for 0, 8, 16, 32, or 64 bytes; each mailbox can be assigned to transmit or receive standard/extended messages.

- up to 6 frames with automatic pointer handling and DMA support.

- abort capability, configurable priority (lowest ID, lowest buffer number, or highest priority).

- 128 slots (8 bytes each), configurable as transmitter or receiver.

- Programmable clock source, supports peripheral clock or oscillator.

- RAM usable for general-purpose storage (not required for transmission/reception).

- Special modes:

  - Listen-Only Mode (LOM)

  - Loop-Back mode for self-test

  - Pretended networking in low-power modes (Doze and Stop)

- Timing and synchronization:

  - 16-bit free-running timer with optional external time tick

  - Global network time synchronization via specific messages

  - Synchronization indication through SYNCH bit in Error Status1 register

- Error handling:

  - CRC status for transmitted messages

  - Detection and correction of memory read errors using 5 parity bits per byte (corrects single-bit errors, detects two-bit errors)

- Advanced receive filtering:

  - ID filtering supports 128 extended IDs, 256 standard IDs,or 512 partial (8-bit) IDs

  - Up to 32 elements in ID Filter Table

  - Supports Identifier Acceptance Filter Hit Indicator (IDHIT)

- Transceiver Delay Compensation (TDC) for CAN-FD high-speed transmission.

- Low latency for high-priority messages via arbitration.

- Interrupts: maskable and independent per mailbox/FIFO.

- Fully backward compatible with previous CAN-FD versions.

Table 2-14: CAN Pin Descriptions

| Pin # | Signal Name | Description | Direction | Pin Type |
| --- | --- | --- | --- | --- |
| 145 | CAN2_TX | CAN #2 Transmit | Output | CMOS – 3.3V |
| 143 | CAN2_RX | CAN #2 Receive | Input | CMOS – 3.3V |

    - Inter-Chip Communication (I2C)

| Standard | Notes |
| --- | --- |
| I2C bus specification version 2.1 | Not support Hardware general call support, 10-bit slave addressing, CBUS compatibility |

The I2C bus enables seamless communication between the I2C unit and various external I2C peripherals or microcontrollers. Its simple hardware design provides an efficient and cost-effective method for transferring control and status information between on-chip and off-chip devices.

The I2C bus interface resides on the peripheral bus and supports:

- Data transfer via a buffered interface for reliable communication

- Control and status management through memory-mapped registers

The I2C controller supports the following operating modes:

- Master – Standard-mode (up to 100 Kbit/s), Fast-mode (up to 400 Kbit/s), High-Speed-mode (up to 3.3 Mbit/s).

- Slave – Standard-mode (up to 100 Kbit/s), Fast-mode (up to 400 Kbit/s), High-Speed-mode (up to 3.4 Mbit/s).

Note: 1. In High-Speed Master Mode, operational frequency is limitedby the value of the pull-up resistors on the bus.

2. The SCL frequency f is inversely proportional to the pull-up resistor R

Table 2-15: I2C Pin Descriptions

| Pin # | Signal Name | Description | Direction | Pin Type |
| --- | --- | --- | --- | --- |
| 185 | I2C0_SCL | General I2C 0 Clock. 4.7kΩ pull-up to 3.3V on module. | Bidir | Open Drain – 3.3V |
| 187 | I2C0_SDA | General I2C 0 Data. 4.7kΩ pull-up to 3.3V on the module. | Bidir | Open Drain – 3.3V |
| 189 | I2C3_SCL | General I2C 3 Clock. 4.7kΩ pull-up to 3.3V on the module. | Bidir | Open Drain – 3.3V |
| 191 | I2C3_SDA | General I2C 3 Data. 4.7kΩ pull-up to 3.3V on the module. | Bidir | Open Drain – 3.3V |
| 232 | I2C1_SCL | General I2C 2 Clock. 4.7kΩ pull-up to 1.8V on the module. | Bidir | Open Drain – 1.8V |
| 234 | I2C1_SDA | General I2C 2 Data. 4.7kΩ pull-up to 1.8V on the module. | Bidir | Open Drain – 1.8V |
| 213 | I2C5_SCL | Camera I2C Clock. 4.7kΩ pull-up to 3.3V on the module. | Bidir | Open Drain – 3.3V |
| 215 | I2C5_SDA | Camera I2C Data. 4.7kΩ pull-up to 3.3V on the module. | Bidir | Open Drain – 3.3V |

    - Inter-IC Sound (I2S)

Standard

Inter-IC Sound (I2S) specification

The K3-CoM260 supports 2x Full-Duplex l2S Interfaces.

Features:

- Support full-duplex operation with simultaneous playbaack and recording.

- Compliance with the standard I2S audio format.

- PCM mode with short (one bit-clock wide) and long-fsync (two bit-clock wide) in both master and slave modes.

- Fixed audio parameters:

  - Sampling rate: 48 kHz

  - Data depth: 16 bits

  - Channels: 2 (stereo)

- Configurable system clock (sysclk) modes: 64fs, 128fs,or 256fs.

Table 2-16: I2S Pin Descriptions

| Pin # | Signal Name | Description | Direction | Pin Type |
| --- | --- | --- | --- | --- |
| 199 | I2S0_CLK | I2S Audio Port 0 Clock | Bidir | CMOS – 1.8V |
| 197 | I2S0_LRCK | I2S Audio Port 0 Left/Right Clock | Bidir | CMOS – 1.8V |
| 193 | I2S0_SDOUT | I2S Audio Port 0 Data Out | Output | CMOS – 1.8V |
| 195 | I2S0_SDIN | I2S Audio Port 0 Data In | Input | CMOS – 1.8V |
| 226 | I2S3_SCLK | I2S Audio Port 3 Clock | Bidir | CMOS – 1.8V |
| 224 | I2S3_FS | I2S Audio Port 3 Left/Right Clock | Bidir | CMOS – 1.8V |
| 220 | I2S3_DOUT | I2S Audio Port 3 Data Out | Output | CMOS – 1.8V |
| 222 | I2S3_DIN | I2S Audio Port 3 Data In | Input | CMOS – 1.8V |

    - Gigabit Ethernet

| Standard | Notes |
| --- | --- |
| Gigabit Ethernet (GbE) | IEEE 802.3-2015 |

The on-module Ethernet controller supports:

- 10/100/1000 Gigabit Ethernet

Table 2-17: Gigabit Ethernet Pin Descriptions

| Pin # | Signal Name | Description | Direction | Pin Type |
| --- | --- | --- | --- | --- |
| 184 | PHY1_MDI0- | GbE Transformer Data 0– | Bidir | MDI |
| 186 | PHY1_MDI0+ | GbE Transformer Data 0+ | Bidir | MDI |
| 190 | PHY1_MDI1- | GbE Transformer Data 1– | Bidir | MDI |
| 192 | PHY1_MDI1+ | GbE Transformer Data 1+ | Bidir | MDI |
| 196 | PHY1_MDI2- | GbE Transformer Data 2– | Bidir | MDI |
| 198 | PHY1_MDI2+ | GbE Transformer Data 2+ | Bidir | MDI |
| 202 | PHY1_MDI3- | GbE Transformer Data 3– | Bidir | MDI |
| 204 | PHY1_MDI3+ | GbE Transformer Data 3+ | Bidir | MDI |
| 188 | PHY1_LED1/CFG_LDO0 | Ethernet Link LED (Green) | Output | - |
| 194 | PHY1_LED2/CFG_LDO1 | Ethernet Activity LED (Yellow) | Output | - |

    - Fan

The K3-CoM260 includes a Pulse Width Modulator (PWM) and Tachometer functionality to enable fan control as part of a thermal solution. The PWM driver operates as an open-drain driver and can be configured with normal or inverse polarity. Register selects the PWM high clock (360kHz) or low (360/256 = 1.4kHz).

    - MultiMedia Card (MMC)

The MMC interface functions as a host controller for the MMC bus, enabling data transfer between the external MMC card and the internal system bus master.

Features:

- Compliant with 4-bit SD 3.0 UHS-I specification.

- Compatible with SDHCI register set, with additional vendopr-specific registers.

- Supports 1-bit / 4-bit SD storage

- Supports the following data transfer types as defined in the SDHCI specification:

  - PIO

  - SDMA

  - ADMA

  - ADMA2

- Supports the following speed modes as defined in the SD3.0 specification:

  - Default Speed: up to 12.5 MB/s bandwidth, 3.3 V signal voltaage

  - High Speed: up to 25 MB/s bandwidth, 3.3 V signal voltage

  - SDR12: up to 25 MHz clock, 1.8 V signal voltage

  - SDR25: up to 50 MHz clock, 1.8 V signal voltage

  - SDR50: up to 100 MHz clock, 1.8 V signal voltage

  - SDR104: up to 208 MHz clock, 1.8 V signal voltage

  - DDR50: up to 50 MHz clock, 1.8 V signal voltage

- Supports hardware generation of all command and data transactions, with CRC validation.

- Supports read-wait control and suspend/resume functions for SD/MMC cards.

- Equipped with a 1024-byte data FIFO buffer (2 x 512-byte data blocks).

Table 2-18: MMC Pin Descriptions

| Pin # | Signal Name | Description | Direction | Pin Type |
| --- | --- | --- | --- | --- |
| 238 | MMC2_CLK | SDIO Clock or GPIO #126 | Output | CMOS – 1.8V |
| 218 | MMC2_CMD | SDIO Command or GPIO #127 | Bidir | CMOS – 1.8V |
| 216 | MMC2_DAT0 | SDIO DATA 0 or GPIO #125 | Bidir | CMOS – 1.8V |
| 211 | MMC2_DAT1 | SDIO DATA 1 or GPIO #124 | Bidir | CMOS – 1.8V |
| 206 | MMC2_DAT2 | SDIO DATA 2 or GPIO #123 | Bidir | CMOS – 1.8V |
| 118 | MMC2_DAT3 | SDIO DATA 3 or GPIO #122 | Bidir | CMOS – 1.8V |

Chapter 3. Power and System Management

Table 3-1: Power and System Control Pin Descriptions

| Pin # | Signal Name | Description | Direction | Pin Type |
| --- | --- | --- | --- | --- |
| 251 252 253 254 255 256 257 258 259 260 | VDD_IN | Main power – Supplies PMIC and other registers | Input | 5V |
| 235 | PMIC_BBAT | Real Time Clock. Optionally used to provide back- up power for the RTC in the Power Sequencer. Connects to a Lithium Cell or similar power source. The cell sources power for the RTC when system is disconnected from power. Note: This pin is input only and rechargeable cells or devices such as Super Caps cannot be supported. | Input | 1.85V to 5.5V |
| 214 | FORCE_RECOVERY | Force Recovery strap pin. Held low when SYS_RESET goes high (i.e., during power-on) places system in USB recovery mode. | Input | CMOS – 1.8V |
| 240 | SLEEP/WAKE | Configured as GPIO for optional use to indicate the system should enter or exit sleep mode. | Input | CMOS – 5.0V |
| 233 | SHUTDOWN_REQ | When driven/pulled low by the module, requests the carrier board to shut down. ~5kΩ pull-up to VDD_IN on the module. | Output | Open Drain, VDD_IN |
| 237 | POWER_EN | Signal for module on/off: high level on, low level off. Connects to module power sequencer/PMIC power on/off control input through converter logic. POWER_EN is routed to a Schmitt trigger buffer on the module. A 100kΩ pulldown is also on the module. | Input | Analog 5.0V |
| 239 | SYS_RESET | Module Reset. Reset to the module when driven low by the carrier board. Used as carrier board supply enable when driven high by the module when module power sequence is complete. Used to ensure proper power on/off sequencing between module and carrier board supplies. 10kΩ pull-up to VDD_1V8 on the module. | Bidir | Open Drain, 1.8V |
| 178 | MOD_SLEEP | Module Sleep. When active (low), indicates module has gone to Sleep mode. | Output | CMOS – 1.8V |
| 210 | CLK_32K_OUT | Sleep/Suspend clock. 2.2kΩ pull-up to VDD_1V8 on the module. If used on the carrier board, a buffer should be implemented close to the module pin. If not used, leave NC. | Bidir | CMOS – 1.8V |
| 217 | MODULE_ID | When tied to GND, the module is a legacy type supporting only 5V on VDD_IN. When floating, the module is an advanced type supporting 5V to 20V on VDD_IN. | - | Strap |
| 239 | SYS_RESET | Module Reset. Reset to the module when driven low by the carrier board. Used as carrier board supply enable when driven high by the module when module power sequence is complete. Used to ensure proper power on/off sequencing between module and carrier board supplies. 10kΩ pull-up to VDD_1V8 on the module. | Bidir | Open Drain, 1.8V |
| 178 | MOD_SLEEP | Module Sleep. When active (low), indicates module has gone to Sleep mode. | Output | CMOS – 1.8V |
| 210 | CLK_32K_OUT | Sleep/Suspend clock. 2.2kΩ pull-up to VDD_1V8 on the module. If used on the carrier board, a buffer should be implemented close to the module pin. If not used, leave NC. | Bidir | CMOS – 1.8V |
| 217 | MODULE_ID | When tied to GND, the module is a legacy type supporting only 5V on VDD_IN. When floating, the module is an advanced type supporting 5V to 20V on VDD_IN. | - | Strap |

  - Power Rails

VDD_IN must be supplied by the carrier board that the K3-CoM260 is designed to connect to. All K3-CoM260 interfaces are referenced to on-module voltage rails and no I/O voltage is required to be supplied to the module.

  - Power Management Controller (PMC)

  - Resets

If reset is asserted, the K3-CoM260 SoC and onboard storage is reset. This signal is also used for baseboard power sequencing.

  - VCC_RTC

An optional back up battery can be attached to the VCC_RTC module input to maintain the module real-time clock (RTC) when VDD_IN is not present. Batteries can be used to power the pin, but charging is not supported. This pin is connected directly to the onboard PMIC. RTC accuracy is 20 ppm.

Table 3-2: PMIC_BBAT Pin Descriptions

| Pin # | Signal Name | Description | Direction | Pin Type |
| --- | --- | --- | --- | --- |
| 235 | PMIC_BBAT | Real Time Clock. Optionally used to provide back-up power for the RTC in the Power Sequencer. Connects to a Lithium Cell or similar power source. The cell sources power for the RTC when system is disconnected from power. Power consumption on battery backup operation is 0.1 to 1 μA. | Input | 1.85V to 5.5V |

  - Power Sequencing

K3-CoM260 is required to be powered on and off in a known sequence. Sequencing is determined through a set of control signals; the SYS_RESET signal (when deasserted) is used to indicate when the carrier board can power on. The following sections provide an overview of the power sequencing steps between the carrier board and K3-CoM260. The K3-CoM260 and the product carrier board must be power sequenced properly to avoid potential damage to components on either the module or the carrier board system.

    - Power Up

Note: I/O pins cannot be high (>0.5V) before SYS_RESET goes high. When SYS_RESET is low, the maximum voltage applied to any I/O pin is 0.5V.

During power up, the carrier board must wait until the signal SYS_RESET is deasserted from the K3-CoM260 module before enabling its power; the K3-CoM260 module will deassert the SYS_RESET signal to enable the complete system to boot.

    - Power Down

Shutdown events can be triggered by either the module or the baseboard, but the shutdown event will always be serviced by the baseboard. To do so, the baseboard deasserts POWER_EN, which begins the shutdown power sequence on the module. If the module needs to request a shutdown event in the case of thermal, software, or under-voltage events, it will assert SHUTDOWN_REQ. When the baseboard sees low SHUTDOWN_REQ, it should deassert POWER_EN as soon as possible.

Once POWER_EN is deasserted, the module will assert SYS_RESET, and the baseboard may shut down. SoC 3.3V I/O must reach 0.5V or lower at most 1.5ms after SYS_RESET is asserted. SoC

1.8V I/O must reach 0.5V or lower at most 4ms after SYS_RESET is asserted.

  - Power States

The K3-CoM260 operates in three main power modes: OFF, ON, and SLEEP. The module transitions between these states are based on various events from hardware or software. The figure below shows the transitions between these three states.

Figure 3-1: Power State Transition Diagram

OFF

ON

SLEEP

ON EVENT

OFF EVENT

SLEEP EVENT

WAKE EVENT

    - ON State

The ON power state is entered from either OFF or SLEEP states. In this state, the K3-CoM260 module is fully functional and operates normally. An ON event must occur for a transition between OFF and ON states. The only ON EVENT currently used is a low to high transition on the POWER_EN pin. This must occur with VDD_IN connected to a power rail and POWER_EN is asserted (at a logic1). The POWER_EN control is the carrier board indication to the K3-CoM260 module that the VDD_IN power is good. The carrier board should assert this high only when VDD_IN has reached its required voltage level and is stable. This prevents the K3-CoM260 Module from powering up until the VDD_IN power is stable.

    - OFF State

The OFF state is the default state when the system is not powered. It can only be entered from the ON state, through an OFF event. OFF events are listed in the table below.

Table 3-3: OFF State Events

| Event | Details | Preconditions |
| --- | --- | --- |
| HW Shutdown | Set POWER_EN pin to zero for at least 10 µs, the internal PMIC starts the shutdown sequence. | In ON State |
| SW Shutdown | Software initiated shutdown | ON state, software operational |
| Thermal Shutdown | If the internal temperature of the K3-CoM260 module reaches an unsafe temperature, the hardware is designed to initiate a shutdown. | Any power state |

Note: HW shutdown, SW shutdown, and Thermal shutdown all assert SHUTDOWN_REQ low. System on Module does not initiate power supply shutdown sequence until POWER_EN is deasserted.

SLEEP State

The SLEEP state can only be entered from the ON state. This state allows the module to quickly resume to an operational state without performing a full boot sequence. The SLEEP state also includes a low power mode where the module operates only with enough circuitry powered to allow the device to resume and re-enter the ON state. During this state the output signals from the module are maintained at their logic level prior to entering the state (i.e., they do not change to a 0V level). To exit the SLEEP state a WAKE event must occur; WAKE events can occur from within the module or from external devices through various pins on the module connector.

Table 3-4: SLEEP and WAKE Events

| Event | Details |
| --- | --- |
| Thermal Condition | If the module internal temperature exceeds programmed hot and cold limits the system is forced to wake up, so it can report and take appropriate action (shut down for example). |
| USB VBUS detection | If VBUS is applied to the system (USB cable attached) then the device can be configured to Wake and enumerate. |
| Module connector Interface WAKE signals | Programmable signals on the module connector. |

  - Thermal and Power Monitoring

The K3-CoM260 is designed to operate under various workloads and environmental conditions. It has been designed so that an active or passive heat sink solution can be attached. The module contains various methods through hardware and software to limit the internal temperature to within operating limits.

Chapter 4. Pin Definitions

The function(s) for each pin on the module is fixed to a single Special-Function I/O (SFIO) or software-controlled General Purpose I/O (GPIO). The K3-CoM260 has multiple dedicated GPIOs and each GPIO is individually configurable as Output/Input/Interrupt sources with level/edge controls. SFIO and GPIO functionality is configured using Multi-Purpose I/O (MPIO) pads with each MPIO pad consisting of:

- An output driver with tristate capability, drive strength controls and push-pull mode, open- drain mode, or both.

- An input receiver with either Schmitt mode, CMOS mode, or both.

- A weak pull-up and a weak pull-down.

  - Power-on Reset Behavior

Each MPIO pad has a deterministic power-on reset (PoR) state. The reset state for each pad is chosen to minimize the need of additional on-board components; for example, on-chip weak pull- ups are enabled during PoR for pads which are usually used to drive active-low chip selects eliminating the need for additional pull-up resistors.

The following list is a simplified description of the boot process focusing on those aspects which relate to the MPIO pins:

    - System-level hardware executes the power-up sequence. This sequence ends when system- level hardware releases SYS_RESET_N.

    - The Boot ROM begins executing and programs the on-chip I/O controllers to access the secondary boot device (QSPI).

    - The Boot ROM fetches the Boot Configuration Table (BCT) and boot loader from the secondary boot device.

    - If the BCT and boot loader are fetched successfully, the boot ROM transfers control to the boot loader.

    - Otherwise, the Boot ROM enters USB recovery mode.

  - Sleep Behavior

Sleep is an ultra-low-power standby state in which the module maintains much of its I/O state while most of the chip is powered off. During deep sleep most of the pads are put in a state called Deep Power Down (DPD). The sequence for entering DPD is same across pads.

MPIO pads can vary during deep sleep. They differ regarding:

    - Input buffer behavior during deep sleep

      - Forcibly disabled OR

      - Enabled for use as a GPIO wake event, OR

      - Enabled for some other purpose (e.g., a clock request pin)

    - Output buffer behavior during deep sleep

      - Maintain a static programmable (0, 1, or tristate) constant value OR

      - Capable of changing state (i.e., dynamic while the chip is still in deep sleep)

    - Weak pull-up/pull-down behavior during deep sleep

      - Forcibly disabled OR

      - Can be configured

    - Pads that do not enter deep sleep

      - Some of the pads whose outputs are dynamic during deep sleep are of special type and do not enter deep sleep.

  - GPIO

The K3-CoM260 has multiple dedicated GPIOs. Each GPIO can be individually configurable as an Output, Input, or Interrupt source with level/edge controls. The pins listed in the following table are dedicated GPIOs; some with alternate SFIO functionality. Many other pins not included in this list are capable of being configured as GPIOs instead of the SFIO functionality the pin name suggests (e.g., UART, SPI, I2S, etc.). All pins that can support GPIO functionality have this exposed in the Pinmux.

Table 4-1: GPIO Pin Descriptions

| Pin # | Signal Name | Description | Direction | Pin Type |
| --- | --- | --- | --- | --- |
| 87 | USB0_VBUS_DET | GPIO #58 or USB 0 VBUS Enable #0 | Bidir | CMOS – 1.8V |
| 118 | MMC2_DAT3 | GPIO #122 or SDIO DATA 3 | Bidir | CMOS – 1.8V |
| 124 | BT_M2_WAKE_AP | GPIO #77 | Bidir | CMOS – 3.3V |
| 126 | BT_M2_EN | GPIO #57 | Bidir | CMOS – 1.8V |
| 127 | PWR_LED_CTRL | GPIO #59 | Bidir | CMOS – 1.8V |
| 128 | W_DISABLE1_CTRL | GPIO #73 | Bidir | CMOS – 1.8V |
| 130 | CAM_MUX_SEL | GPIO #103 | Bidir | CMOS – 1.8V |
| 206 | MMC2_DAT2 | GPIO #123 or SDIO DATA 2 | Bidir | CMOS – 1.8V |

| Pin # | Signal Name | Description | Direction | Pin Type |
| --- | --- | --- | --- | --- |
| 211 | MMC2_DAT1 | GPIO #124 or SDIO DATA 1 | Bidir | CMOS – 1.8V |
| 212 | M2_ALERT | GPIO #56 | Bidir | CMOS – 1.8V |
| 216 | MMC2_DAT0 | GPIO #125 or SDIO DATA 0 | Bidir | CMOS – 1.8V |
| 218 | MMC2_CMD | GPIO #126 or SDIO Command | Bidir | CMOS – 1.8V |
| 228 | MMC2_CLK | GPIO #127 or SDIO Clock | Bidir | CMOS – 1.8V |

Note: For cell shading, light green indicates ground; light blue indicates K3-CoM260 specific functionality.

Pin List

| K3 Signal Name | K3-CoM260 Function | Pin # Top Odd |  | Pin # Bottom Even | K3 Signal Name | K3-CoM260 Function |
| --- | --- | --- | --- | --- | --- | --- |
| GND | GND | 1 |  | 2 | GND | GND |
| MIPI_CSI0_DN2 | MIPI_CSI0_DN2 | 3 |  | 4 | MIPI_CSI0_DN0 | MIPI_CSI0_DN0 |
| MIPI_CSI0_DP2 | MIPI_CSI0_DP2 | 5 |  | 6 | MIPI_CSI0_DP0 | MIPI_CSI0_DP0 |
| GND | GND | 7 |  | 8 | GND | GND |
| MIPI_CSI1_CLKN | MIPI_CSI1_CLKN | 9 |  | 10 | MIPI_CSI0_CLKN | MIPI_CSI0_CLKN |
| MIPI_CSI1_CLKP | MIPI_CSI1_CLKP | 11 |  | 12 | MIPI_CSI0_CLKP | MIPI_CSI0_CLKP |
| GND | GND | 13 |  | 14 | GND | GND |
| MIPI_CSI0_DN3 | MIPI_CSI0_DN3 | 15 |  | 16 | MIPI_CSI0_DN1 | MIPI_CSI0_DN1 |
| MIPI_CSI0_DP3 | MIPI_CSI0_DP3 | 17 |  | 18 | MIPI_CSI0_DP1 | MIPI_CSI0_DP1 |
| GND | GND | 19 |  | 20 | GND | GND |
| MIPI_CSI2_DN2 | MIPI_CSI2_DN2 | 21 |  | 22 | MIPI_CSI2_DN0 | MIPI_CSI2_DN0 |
| MIPI_CSI2_DP2 | MIPI_CSI2_DP2 | 23 |  | 24 | MIPI_CSI2_DP0 | MIPI_CSI2_DP0 |
| GND | GND | 25 |  | 26 | GND | GND |
| MIPI_CSI3_CLKN | MIPI_CSI3_CLKN | 27 |  | 28 | MIPI_CSI2_CLKN | MIPI_CSI2_CLKN |
| MIPI_CSI3_CLKP | MIPI_CSI3_CLKP | 29 |  | 30 | MIPI_CSI2_CLKP | MIPI_CSI2_CLKP |
| GND | GND | 31 |  | 32 | GND | GND |
| MIPI_CSI2_DN3 | MIPI_CSI2_DN3 | 33 |  | 34 | MIPI_CSI2_DN1 | MIPI_CSI2_DN1 |
| MIPI_CSI2_DP3 | MIPI_CSI2_DP3 | 35 |  | 36 | MIPI_CSI2_DP1 | MIPI_CSI2_DP1 |
| GND | GND | 37 |  | 38 | GND | GND |
| USB_DRD_RX1N | USB_DRD_RX1N | 39 |  | 40 | PCIe4/USB3-D_RX0n (PCIe2/USB3-B_RX0n) | PCIe4/USB3-D_RX0n (PCIe2/USB3-B_RX0n) |
| USB_DRD_RX1P | USB_DRD_RX1P | 41 |  | 42 | PCIe4/USB3-D_RX0p (PCIe2/USB3-B_RX0p) | PCIe4/USB3-D_RX0p (PCIe2/USB3-B_RX0p) |
| GND | GND | 43 |  | 44 | GND | GND |
| USB_DRD_TX1N | USB_DRD_TX1N | 45 |  | 46 | PCIE4/USB3-D_TX0N | PCIe4/USB3-D_TX0n (PCIe2/USB3-B_TX0n) |

| K3 Signal Name | K3-CoM260 Function | Pin # Top Odd |  | Pin # Bottom Even | K3 Signal Name | K3-CoM260 Function |
| --- | --- | --- | --- | --- | --- | --- |
| USB_DRD_TX1P | USB_DRD_TX1P | 47 |  | 48 | PCIE4/USB3-D_TX0P | PCIE4/USB3-D_TX0P |
| GND | GND | 49 |  | 50 | GND | GND |
| USB_DRD_RX2N | USB_DRD_RX2N | 51 |  | 52 | PCIE4_REFCLK_N | PCIE4_REFCLK_N |
| USB_DRD_RX2P | USB_DRD_RX2P | 53 |  | 54 | PCIE4_REFCLK_P | PCIE4_REFCLK_P |
| GND | GND | 55 |  | 56 | GND | GND |
| USB_DRD_TX2N | USB_DRD_TX2N | 57 |  | 58 | PCIE3/USB3-C_RX0N | PCIE3/USB3-C_RX0N |
| USB_DRD_TX2P | USB_DRD_TX2P | 59 |  | 60 | PCIE3/USB3-C_RX0P | PCIE3/USB3-C_RX0P |
| GND | GND | 61 |  | 62 | GND | GND |
| EDP1_TX0N | EDP1_TX0N | 63 |  | 64 | PCIE3/USB3-C_TX0N | PCIE3/USB3-C_TX0N |
| EDP1_TX0P | EDP1_TX0P | 65 |  | 66 | PCIE3/USB3-C_TX0P | PCIE3/USB3-C_TX0P |
| GND | GND | 67 |  | 68 | GND | GND |
| EDP1_TX1N | EDP1_TX1N | 69 |  | 70 | MIPI_DSI1_D0N | MIPI_DSI1_D0N |
| EDP1_TX1P | EDP1_TX1P | 71 |  | 72 | MIPI_DSI1_D0P | MIPI_DSI1_D0P |
| GND | GND | 73 |  | 74 | GND | GND |
| EDP1_TX2N | EDP1_TX2N | 75 |  | 76 | MIPI_DSI1_CLKN | MIPI_DSI1_CLKN |
| EDP1_TX2P | EDP1_TX2P | 77 |  | 78 | MIPI_DSI1_CLKP | MIPI_DSI1_CLKP |
| GND | GND | 79 |  | 80 | GND | GND |
| EDP1_TX3N | EDP1_TX3N | 81 |  | 82 | MIPI_DSI1_D1N | MIPI_DSI1_D1N |
| EDP1_TX3P | EDP1_TX3P | 83 |  | 84 | MIPI_DSI1_D1P | MIPI_DSI1_D1P |
| GND | GND | 85 |  | 86 | GND | GND |
| GPIO58 | USB0_VBUS_DET | 87 |  | 88 | GPIO74 | CLK_CAMCK2 |
| SPI0_MOSI | SPI0_MOSI | 89 |  | 90 | GPIO63 | CLK_CAMCK4 |
| SPI0_SCK | SPI0_SCK | 91 |  | 92 | GPIO53 | MIPI_CSI3_PWDN |
| SPI0_MISO | SPI0_MISO | 93 |  | 94 | GPIO02 | MIPI_CSI1_PWDN |
| SPI0_CS | SPI0_CS | 95 |  | 96 | e/DP1_HPD | e/DP1_HPD |
| SPI0_CS1 | SPI0_CS1 | 97 |  | 98 | EDP1_AUXN | EDP1_AUXN |
| UART6_TXD | UART6_TXD | 99 |  | 100 | EDP1_AUXP | EDP1_AUXP |
| UART6_RXD | UART6_RXD | 101 |  | 102 | GND | GND |
| UART6_RTS | UART6_RTS | 103 |  | 104 | R-SPI0_MOSI | R-SPI0_MOSI |
| UART6_CTS | UART6_CTS | 105 |  | 106 | R-SPI0_SCK | R-SPI0_SCK |
| GND | GND | 107 |  | 108 | R-SPI0_MISO | R-SPI0_MISO |
| USB20_A_DRD_USB_M | USB20_A_DRD_USB_M | 109 |  | 110 | R-SPI0_CS | R-SPI0_CS |
| USB20_A_DRD_USB_P | USB20_A_DRD_USB_P | 111 |  | 112 | R-SPI0_CS1 | R-SPI0_CS1 |
| GND | GND | 113 |  | 114 | GPIO45 | MIPI_CSI0_PWDN |

| K3 Signal Name | K3-CoM260 Function | Pin # Top Odd |  | Pin # Bottom Even | K3 Signal Name | K3-CoM260 Function |
| --- | --- | --- | --- | --- | --- | --- |
| USB20_B_USB_M | USB20_B_USB_M | 115 |  | 116 | GPIO75 | CLK_CAMCK1 |
| USB20_B_USB_P | USB20_B_USB_P | 117 |  | 118 | GPIO122 | MMC2_DAT3 |
| GND | GND | 119 |  | 120 | GPIO44 | MIPI_CSI2_PWDN |
| USB20_HOST_M | USB20_HOST_M | 121 |  | 122 | GPIO85 | CLK_CAMCK3 |
| USB20_HOST_P | USB20_HOST_P | 123 |  | 124 | GPIO77 | BT_M2_WAKE_AP |
| GND | GND | 125 |  | 126 | GPIO57 | BT_M2_EN |
| GPIO59 | PWR_LED_CTRL | 127 |  | 128 | GPIO73 | W_DISABLE1_CTRL |
| GND | GND | 129 |  | 130 | GPIO103 | CAM_MUX_SEL |
| PCIE0_RX0N | PCIE0_RX0N | 131 |  | 132 | GND | GND |
| PCIE0_RX0P | PCIE0_RX0P | 133 |  | 134 | PCIE0_TX0N | PCIE0_TX0N |
| GND | GND | 135 |  | 136 | PCIE0_TX0P | PCIE0_TX0P |
| PCIE0_RX1N | PCIE0_RX1N | 137 |  | 138 | GND | GND |
| PCIE0_RX1P | PCIE0_RX1P | 139 |  | 140 | PCIE0_TX1N | PCIE0_TX1N |
| GND | GND | 141 |  | 142 | PCIE0_TX1P | PCIE0_TX1P |
| CAN2_RX | CAN2_RX | 143 |  | 144 | GND | GND |
| CAN2_TX | CAN2_TX | 145 |  | 146 | GND | GND |
| GND | GND | 147 |  | 148 | PCIE1_TX0N | PCIE1_TX0N |
| PCIE1_RX0N | PCIE1_RX0N | 149 |  | 150 | PCIE1_TX0P | PCIE1_TX0P |
| PCIE1_RX0P | PCIE1_RX0P | 151 |  | 152 | GND | GND |
| GND | GND | 153 |  | 154 | PCIE1_TX1N | PCIE1_TX1N |
| PCIE1_RX1N | PCIE1_RX1N | 155 |  | 156 | PCIE1_TX1P | PCIE1_TX1P |
| PCIE1_RX1P | PCIE1_RX1P | 157 |  | 158 | GND | GND |
| GND | GND | 159 |  | 160 | PCIE0_REFCLK_N | PCIE0_REFCLK_N |
| PCIE2/USB3-B_RX0N | PCIE2/USB3-B_RX0N | 161 |  | 162 | PCIE0_REFCLK_P | PCIE0_REFCLK_P |
| PCIE2/USB3-B_RX0P | PCIE2/USB3-B_RX0P | 163 |  | 164 | GND | GND |
| GND | GND | 165 |  | 166 | PCIE2/USB3-B_TX0N | PCIE2/USB3-B_TX0N |
| PCIE5_RX0N | PCIE5_RX0N | 167 |  | 168 | PCIE2/USB3-B_TX0P | PCIE2/USB3-B_TX0P |
| PCIE5_RX0P | PCIE5_RX0P | 169 |  | 170 | GND | GND |
| GND | GND | 171 |  | 172 | PCIE5_TX0N | PCIE5_TX0N |
| PCIE5_REFCLK_N | PCIE5_REFCLK_N | 173 |  | 174 | PCIE5_TX0P | PCIE5_TX0P |
| PCIE5_REFCLK_P | PCIE5_REFCLK_P | 175 |  | 176 | GND | GND |
| GND | GND | 177 |  | 178 | GPIO42 | MOD_SLEEP |
| PCIeA_WAKEn | PCIeA_WAKEn | 179 |  | 180 | PCIeA_CLKREQn | PCIeA_CLKREQn |
| PCIeA_PERSTn | PCIeA_PERSTn | 181 |  | 182 | PCIeE_CLKREQn | PCIeE_CLKREQn |
| PCIeE_PERSTn | PCIeE_PERSTn | 183 |  | 184 | PHY1_MDI0- | PHY1_MDI0- |
| I2C0_SCL | I2C0_SCL | 185 |  | 186 | PHY1_MDI0+ | PHY1_MDI0+ |
| I2C0_SDA | I2C0_SDA | 187 |  | 188 | PHY1_LED1/CFG_LDO0 | PHY1_LED1/CFG_LDO0 |

| K3 Signal Name | K3-CoM260 Function | Pin # Top Odd |  | Pin # Bottom Even | K3 Signal Name | K3-CoM260 Function |
| --- | --- | --- | --- | --- | --- | --- |
| I2C3_SCL | I2C3_SCL | 189 |  | 190 | PHY1_MDI1- | PHY1_MDI1- |
| I2C3_SDA | I2C3_SDA | 191 |  | 192 | PHY1_MDI1+ | PHY1_MDI1+ |
| I2S0_SDOUT | I2S0_SDOUT | 193 |  | 194 | PHY1_LED2/CFG_LDO1 | PHY1_LED2/CFG_LDO1 |
| I2S0_SDIN | I2S0_SDIN | 195 |  | 196 | PHY1_MDI2- | PHY1_MDI2- |
| I2S0_LRCK | I2S0_LRCK | 197 |  | 198 | PHY1_MDI2+ | PHY1_MDI2+ |
| I2S0_CLK | I2S0_CLK | 199 |  | 200 | GND | GND |
| GND | GND | 201 |  | 202 | PHY1_MDI3- | PHY1_MDI3- |
| UART1_TXD | UART1_TXD | 203 |  | 204 | PHY1_MDI3+ | PHY1_MDI3+ |
| UART1_RXD | UART1_RXD | 205 |  | 206 | GPIO123 | MMC2_DAT2 |
| UART1_RTS | UART1_RTS | 207 |  | 208 | FAN_TACH | FAN_TACH |
| UART1_CTS | UART1_CTS | 209 |  | 210 | PMIC_32K_OUT | PMIC_32K_OUT |
| GPIO124 | MMC2_DAT1 | 211 |  | 212 | GPIO56 | M2_ALERT |
| I2C5_SCL | I2C5_SCL | 213 |  | 214 | GPIO69 | FORCE_RECOVERY |
| I2C5_SDA | I2C5_SDA | 215 |  | 216 | GPIO125 | MMC2_DAT0 |
| GND | MODULE_ID | 217 |  | 218 | GPIO126 | MMC2_CMD |
| GPIO03 | PCIeD_PERSTn | 219 |  | 220 | I2S3_SDOUT | I2S3_SDOUT |
| GPIO05 | PCIeD_CLKREQn | 221 |  | 222 | I2S3_SDIN | I2S3_SDIN |
| GPIO12 | PCIeC_PERSTn | 223 |  | 224 | I2S3_LRCK | I2S3_LRCK |
| GPIO14 | PCIeC_CLKREQn | 225 |  | 226 | I2S3_CLK | I2S3_CLK |
| PCIE3_REFCLK_N | PCIE3_REFCLK_N | 227 |  | 228 | GPIO127 | MMC2_CLK |
| PCIE3_REFCLK_P | PCIE3_REFCLK_P | 229 |  | 230 | FAN_PWM | FAN_PWM |
| GND | GND | 231 |  | 232 | I2C1_SCL | I2C1_SCL |
| GPIO88 | SHUTDOWN_REQ | 233 |  | 234 | I2C1_SDA | I2C1_SDA |
| VCC_RTC | VCC_RTC | 235 |  | 236 | UART0_TXD_PMU | UART0_TXD_PMU |
| POWER_EN | POWER_EN | 237 |  | 238 | UART0_RXD_PMU | UART0_RXD_PMU |
| PMIC_RST_OUTn | PMIC_RST_OUTn | 239 |  | 240 | GPIO84 | SLEEP/WAKE |
| GND | GND | 241 |  | 242 | GND | GND |
| GND | GND | 243 |  | 244 | GND | GND |
| GND | GND | 245 |  | 246 | GND | GND |
| GND | GND | 247 |  | 248 | GND | GND |
| GND | GND | 249 |  | 250 | GND | GND |
| VDD_IN | VDD_IN | 251 |  | 252 | VDD_IN | VDD_IN |
| VDD_IN | VDD_IN | 253 |  | 254 | VDD_IN | VDD_IN |
| VDD_IN | VDD_IN | 255 |  | 256 | VDD_IN | VDD_IN |
| VDD_IN | VDD_IN | 257 |  | 258 | VDD_IN | VDD_IN |
| VDD_IN | VDD_IN | 259 |  | 260 | VDD_IN | VDD_IN |

Chapter 5. Electrical, Mechanical, and Thermal Characteristics

  - Operating and Absolute Maximum Ratings

The parameters listed in following table are specific to a temperature range and operating voltage. Operating the K3-CoM260 beyond these parameters is not recommended.

Note: MODULE_ID low is for low-power mode and MODULE_ID high is for high-power mode.

Warning: Exceeding the listed conditions may damage and/or affect long-term reliability of the part. The K3-CoM260 module should never be subjected to conditions extending beyond the ratings listed below

Table 5-1: Recommended Operating Conditions

| Symbol | Parameter | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- |
| VDDDC | VDD_IN (MODULE_ID low) (see note) | 4.75 | 5 | 5.25 | V |
|  | VDD_IN (MODULE_ID high) (see note) | 4.75 | - | 20 | V |
|  | VCC_RTC | 1.85 | - | 5.5 | V |

Absolute maximum ratings describe stress conditions. These parameters do not set minimum and maximum operating conditions that will be tolerated over extended periods of time. If the device is exposed to these parameters for extended periods of time, performance is not guaranteed, and device reliability may be affected. It is not recommended to operate the K3-CoM260 module under these conditions.

Table 5-2: Absolute Maximum Ratings

| Symbol | Parameter | Minimum | Maximum | Unit | Notes |
| --- | --- | --- | --- | --- | --- |
| VDDMAX | VDD_IN (MODULE_ID low) | -0.5 | 5.5 | V |  |
|  | VDD_IN (MODULE_ID high) | -0.5 | 20.5 | V |  |
|  | PMIC_BBAT | -0.3 | 6.0 | V |  |
| IDDMAX | VDD_IN Imax | - | 5 | A |  |
| VM_PIN | Voltage applied to any powered I/O pin | -0.5 | VDD + 0.5 | V | VDD + 0.5V when SYS_RESET is high and associated I/O rail powered. I/O pins cannot be high (>0.5V) before SYS_RESET goes high. When SYS_RESET is low, the maximum voltage applied to any I/O pin is 0.5V |
|  | DD pins configured as open drain | -0.5 | 3.63 | V | The pin’s output-driver must be set to open-drain mode. |
| TOP | Operating Temperature | -25 | 70 | °C |  |
| TSTG | Storage Temperature (ambient) |  |  | °C |  |
| MMAX | Mounting Force | - |  | kgf1 | Maximum force applied to PCB. |
| Note: kgf stands for kilogram-force. |  |  |  |  |  |

  - Digital Logic

Voltages less than the minimum stated value can be interpreted as an undefined state or logic level low which may result in unreliable operation. Voltages exceeding the maximum value can damage and/or adversely affect device reliability.

Table 5-3: CMOS Pin Type DC Characteristics

| Symbol | Description | Minimum | Maximum | Units |
| --- | --- | --- | --- | --- |
| VIL | Input Low Voltage | -0.5 | 0.25 x VDD | V |
| VIH | Input High Voltage | 0.75 x VDD | 0.5 + VDD | V |
| VOL | Output Low Voltage (IOL = 1mA) | - | 0.15 x VDD | V |
| VOH | Output High Voltage (IOH = -1mA) | 0.85 x VDD | - | V |

Table 5-4: Open Drain Pin Type DC Characteristics

| Symbol | Description | Minimum | Maximum | Units |
| --- | --- | --- | --- | --- |
| VIL | Input Low Voltage | -0.5 | 0.2 x VDD | V |
| VIH | Input High Voltage | 0.8 x VDD | 3.63 | V |
| VOL | Output Low Voltage (IOL = 1mA) | - | 0.15 x VDD | V |
|  | I2C [1,0] Output Low Voltage (IOL = 2mA) (see note) | - | 0.3 x VDD | V |
| VOH | Output High Voltage (IOH = -1mA) | 0.7 x VDD | - | V |

Note: I2C[5,3,0]_[SCL, SDA] pins pull-up to 3.3V through on module 4.7kΩ resistor. I2C1_[SCL, SDA] pins pull-up to 1.8V through on module 4.7kΩ resistor.

Environmental and Mechanical Screening

Table 5-5: Environmental Testing

| Test | Conditions | Reference Standard | Results |
| --- | --- | --- | --- |
| Temperature Humidity Biased | 85°C / 85% RH, 168 hours, Power ON | JESD22-A101 |  |
| Temperature Cycling | -40°C to 105°C, 375 cycles, non-operational | JESD22-A104, IPC9701 |  |
| Temp/Humidity Cycle | 25°C to 65°C, 93% RH, six cycles | NV Standard |  |
| Mechanical Shock – 140G Non-Op | 140G, 2 msec, half sine, one shock/orientation, six orientations total, non-operational | JESD22-B110 |  |
| Mechanical Shock – 50G Op | 50G, 6 msec, half sine, three shocks/orientation, six orientations total, operational | IEC 600068 2-27 |  |
| Connector Insertion Cycling | Insert/Withdraw SD card and connector, 30 cycles | EIA-364 |  |
| Random Vibration – 2G Non-Op | 5-500 Hz, 2 Grms, 1 hour/axis, three axes total, non-operational | IEC60068-2-64 |  |
| Random Vibration – 1G Op | 10-500 Hz, 1 Grms, 30 min/axis, three axes total, operational | IEC60068-2-64 |  |
| Hard Boot | Power ON/OFF, ON for 150 sec, OFF for 30 sec 2000 cycles at 25°C 1000 cycles at -5°C 1000 cycles at 45°C | NV Standard |  |
| Operational Low Temp | -5°C, 24 hours, operational | NV Standard |  |

| Test | Conditions | Reference Standard | Results |
| --- | --- | --- | --- |
| Operational High Temp | 45°C, 90%RH, 336 hours, operational | NV Standard |  |
| MTBF / Failure Rate | Controlled Environment (GB), T = 35°C or 50°C, CL = 90% | Telcordia (TelC4) SR-332, ISSUE4 Parts Count (Method 1) |  |
| MTBF / Failure Rate | Uncontrolled Environment (GF) T = 35°C or 50°C, CL = 90% | Telcordia (TelC4) SR-332, ISSUE4 Parts Count (Method 1) |  |

Notes: MTBF numbers below are based on Telcordia (TelC4) SR-332, ISSUE4, Calculation Methodology:

Parts Count (Method 1), UCL = 90%, Quality level: II

- Orin Nano 8GB: GB at 35: 3,911,708 hours GB at 50: 2,107,458 hours GF at 35: 2,888,137 hours GF at 50: 1,468,031 hours.

- Orin Nano 4GB: GB at 35: 4,558,489 hours GB at 50: 2,250,633 hours GF at 35: 3,210,552 hours GF at 50: 1,535,375 hours.

Storage and Handling

Note: Transportation is a limited range of time that is covered by AEC grade 3 specs (-40°C to 85°C). Longer term storage at hubs, distribution points, and warehousing where climate controls are in place should follow conditions mentioned above.

Duration based on product being packed and stored in a controlled environment without power on.

Table 5-6: Typical Handling and Storage Environment

| Parameter | Description |
| --- | --- |
| Storage temperature (ambient)1 | 18°C to 30°C |
| Storage humidity | 30% to 70% RH |
| Storage life2 | 5 years from NVIDIA shipment date to customers |

  - Drawing and Dimensions

Module Size: 69.6 mm x 45 mm

Note:

- All dimensions are in millimeters unless otherwise specified.

- Tolerances are: .X ± 0.25, XX ± is 0.1, Angle ± is 1.

Module Weight: 0.028kg

Figure 5-1: Top View

Figure 5-2: Bottom View